D.I.Y. Guide

Thank you for choosing exaU2I for your very special high-end audio project. We hope you will enjoy it as much as we enjoyed designing, building and using it in our own D.I.Y audio systems.

Please be careful – exaU2I can be destroyed by static, shortcuts, …

Quick Start

The fist thing to do probably is to put exaU2I to a very quick test. You can install an ASIO capable audio player, the ASIO exaU2I driver, connect the USB cable and play something. You will see the power light on. The FIFO light will come on when you start playback. One of the sync LEDs will indicate the sampling frequency set by your player.

You can find step by step instructions how to setup an ASIO player and driver in the following sections of this document.

When you are done with the quick test, you will know that your music source is working and exaU2I is behaving as expected. At this point you can look at the schematics and connect exaU2I to your DAC or DACs.

The only special note to make is that you have to use the DAC power supply for the exaU2I outputs. More specifically, you have to use the power supply that powers the digital inputs of your DAC(s). The exaU2I output circuits are galvanically isolated from the rest of the board. They operate as a part of your DAC.

Setup and Configuration

1. ASIO Driver Installation

The first step required in the setup process is to install the ASIO driver for exaU2I:

Don’t connect the exaU2I device to the PC until the driver is installed. Following this rule makes troubleshooting easier. Don’t worry if you’ve connected the device already. Just disconnect it.

2. Download the latest driver from this website.  Make sure your antivirus software is running before you start the download. Do not use divers downloaded from other websites.

Double-click on Install.exe to start the installation. Follow the instructions. In most cases you just need to accept the defaults on the following screens.

Connect the exaU2I device. You should see the PWR LED on. You should see a pop-up message on the computer screen indicating that exaU2I device is successfully installed.

2. Configuring Audio Player with ASIO Output

You can use any ASIO-enabled software to stream PCM audio to exaU2I. In our experience

Foobar, J. River Media Center and AlbumPlayer provided the best performance and sound quality. Other PC audio software that can be used include Winamp and MediaMonkey. There is an ASIO output plug-in for Windows Media Player; unfortunately we were unable to make it work. You can find information on the Internet about installing and configuring these applications for use with ASIO drivers.

See Appendix A at the end of this document for step by step instructions for installing Foobar2000 and configuring the Foobr2000 ASIO output plug-in.

3. Connecting exaU2I to Your DAC Kit(s)

exaU2I will work with most DIY DAC boards with I2S input. For our own projects we use a DAC kit based on the ESS Technology ES9018 Sabre32 Reference chip. At the present time ES9018 is the only chip that we were able to drive at 352.8 kHz and 384 kHz. Let us know about experience with other DACs.

Note: The length of the connection cables should be less then 100mm.