1. Organic Semiconductor Solid Electrolytic Capacitors

| Classification | Series | Applications | Operating Temperature Range (ĄÉ) | Load Life Time (hours) | Voltage Range (VDC) | Capacitance Range (§Ţ) |
| --- | --- | --- | --- | --- | --- | --- |
| Lead Type | FX | Standard, With Organic Semiconductor (APRO-CAP) | -55 ~ 105 | 2000 | 6.3 ~ 30 | 1.0 ~ 3300 |
|  | FD | Miniaturized, With Organic Semiconductor (APRO-CAP) | -55 ~ 105 | 2000 | 2 ~ 25 | 33 ~ 2200 |
|  | FE | Height 5mm, With Organic Semiconductor (APRO-CAP) | -55 ~ 105 | 2000 | 4 ~ 25 | 1.0 ~ 150 |
|  | FH | Long life, With Organic Semiconductor (APRO-CAP) | -55 ~ 105 | 5000 | 6.3 ~ 25 | 4.7 ~ 330 |

2. Conducting Polymer Aluminum Electrolytic Capacitors

| Classification | Series | Applications | Operating Temperature Range (ĄÉ) | Load Life Time (hours) | Voltage Range (VDC) | Capacitance Range (§Ţ) |
| --- | --- | --- | --- | --- | --- | --- |
| Lead Type | FB | Lead type, With Conducting Polymer (Hi-CAP) | -55 ~ 105 | 1000 | 4 ~ 16 | 27 ~ 820 |
| Chip Type | FA | Winding Chip type, With Conducting Polymer (Hi-CAP) | -55 ~ 105 | 1000 | 4 ~ 16 | 27 ~ 470 |
|  | FC | Chip type, With Conducting Polymer (Hi-CAP) | -40 ~ 105 | 1000 | 6.3 ~ 16 | 2.2 ~ 47 |

3. Surface Mount Aluminum Electrolytic Capacitors

| Classification | Series | Applications | Operating Temperature Range (ĄÉ) | Load Life Time (hours) | Voltage Range (VDC) | Capacitance Range (§Ţ) |
| --- | --- | --- | --- | --- | --- | --- |
| 85ĄÉ Product | SC | 5.5mm Height, Standard | -40 ~ 85 | 2000 | 4 ~ 50 | 0.1 ~ 150 |
| Non-Polarized | NC | 5.5mm Height, Non-Polarized | -40 ~ 85 | 2000 | 6.3 ~ 50 | 0.1 ~ 47 |
|  | CN | 5.5mm Height, Non-Polarized, Wide Temp Range | -55 ~ 105 | 1000 | 6.3 ~ 50 | 0.1 ~ 47 |
| 105ĄÉ Product | RC | 5.5mm Height, Wide Temp Range | -55 ~ 105 | 1000 | 6.3 ~ 50 | 0.1 ~ 330 |
|  | JC | 6mm Height, Wide Temp Range | -55 ~ 105 | 2000 | 4 ~ 50 | 0.1 ~ 100 |
|  | TC | High CV, Wide Temp Range | -55 ~ 105 | 2000 | 6.3 ~ 50 | 22 ~ 1000 |
| Low Impedance | ZC | 5.5mm Height, Low Impednce | -55 ~ 105 | 1000 | 6.3 ~ 35 | 1 ~ 100 |
|  | CK | 6mm Height, Low Impednce | -55 ~ 105 | 2000 | 6.3 ~ 50 | 1 ~ 330 |
|  | AC | 6mm Height, Long Life | -40 ~ 105 | 5000 | 4 ~ 50 | 0.1 ~ 100 |
| 125ĄÉ Product | UC | 125ĄÉ Product, Wide Temp Range | -40 ~ 125 | 2000 | 6.3 ~ 50 | 0.1 ~ 330 |
| Low Leakage | CL | Low Leakage Current | -40 ~ 85 | 2000 | 6.3 ~ 50 | 0.1 ~ 150 |

4. Miniature Aluminum Electrolytic Capacitors

| Classification | Series | Applications | Operating Temperature Range (ĄÉ) | Load Life Time (hours) | Voltage Range (VDC) | Capacitance Range (§Ţ) |
| --- | --- | --- | --- | --- | --- | --- |
| 85ĄÉ General Use | SD | Standard | -40(-25) ~ 85 | 2000 | 6.3 ~ 500 | 1.0 ~ 22000 |
|  | SV | Low Profile Sized | -40 ~ 85 | 2000 | 6.3 ~ 50 | 15 ~ 4700 |
|  | SS | 7mm Height, Standard | -40 ~ 85 | 2000 | 4 ~ 63 | 0.1 ~ 220 |
|  | SE | 5mm Height, Standard | -40 ~ 85 | 2000 | 4 ~ 63 | 0.1 ~ 150 |
| 105ĄÉ General Use | RD | Standard, Wide Tmperature Range | -55(-40,-25) ~105 | 1000~2000 | 6.3 ~ 450 | 2.2 ~ 22000 |
|  | RV | Low Profile Sized, Wide Tmperature Range | -55 ~ 105 | 1000 | 6.3 ~ 50 | 22 ~ 4700 |
|  | RK | 7mm Heigt, Wide Tmperature Range | -55 ~ 105 | 1000 | 4 ~ 63 | 0.1 ~ 68 |
|  | RE | 5mm Heigt, Wide Tmperature Range | -55 ~ 105 | 1000 | 4 ~ 50 | 0.1 ~ 220 |
| Low Impedance | RZ | Extremely Low Impedance | -55 ~ 105 | 2000~5000 | 6.3 ~ 63 | 1.0 ~ 15000 |
|  | WD | Extremely Low Impedance, Miniaturized | -55 ~ 105 | 2000~5000 | 6.3 ~ 50 | 10 ~ 15000 |
|  | WL | Extremely Low Impedance, Miniaturized, Wide Voltage | -40 ~ 105 | 2000~5000 | 6.3 ~ 450 | 0.22 ~ 15000 |
|  | RP | Extremely Low Impedance, Long Life | -55 ~ 105 | 4000~10000 | 6.3 ~ 50 | 1.0 ~ 15000 |
|  | WF | Extremely Low Impedance, Miniaturized, Long Life | -40 ~ 105 | 5000~10000 | 6.3 ~ 100 | 0.47 ~ 15000 |
|  | RQ | Extremely Low Impedance | -55 ~ 105 | 2000~5000 | 6.3 ~ 50 | 4.7 ~ 15000 |
|  | WB | Ultra Low Impedance | -40 ~ 105 | 2000~5000 | 6.3 ~ 100 | 0.22 ~ 15000 |
|  | WZ | Ultra Low Impedance, Miniaturized | -40 ~ 105 | 2000 | 6.3 ~ 16 | 470 ~ 3300 |
|  | WS | Height 7mm, Low Impedance | -55 ~ 105 | 1000 | 6.3 ~ 35 | 6.8 ~100 |
|  | WE | Height 5mm, Low Impedance | -55 ~ 105 | 1000 | 6.3 ~ 35 | 1.0 ~ 100 |
| High Reliability | RS | Long Life (7000hours) | -55 ~ 105 | 5000 ~ 7000 | 10 ~ 63 | 4.7 ~ 4700 |
|  | RW | High Temperature Range, For 125ĄÉ Use | -55(-40) ~ 125 | 1000 ~ 2000 | 10 ~ 250 | 0.47 ~ 470 |
|  | RB | High Temperature Range, For 125ĄÉ Use, Minaturized | -55 ~ 125 | 2000 | 6.3 ~ 50 | 1.0 ~ 15000 |
| Non-Polarized | RN | Wide Temperature Range, Non-Polarized | -40 ~ 105 | 1000 | 6.3 ~ 100 | 0.1 ~ 10000 |
|  | NP | Standard, Non-Polarized | -40 ~ 85 | 2000 | 6.3 ~ 250 | 0.47 ~ 10000 |
|  | NS | Height 7mm, Non-Polarized | -40 ~ 85 | 2000 | 6.3 ~ 63 | 0.1 ~ 47 |
|  | NE | Height 5mm, Non-Polarized | -40 ~ 85 | 1000 | 6.3 ~ 50 | 0.1 ~ 47 |
|  | BP | For Crossover Networks | -40 ~ 85 | 2000 | 25,50,100,200 | 1.0 ~ 100 |
|  | BR | For Crossover Networks, Wide Temp | -40 ~ 105 | 2000 | 200 | 3.3 ~ 100 |
|  | NF | For Horizontal Ddeflection Current Correction | -40 ~ 85 | 1000 | 25,50 | 1.0 ~ 10 |
|  | NH | For Horizontal Deflection Current Correction | -55 ~ 105 | 1000 | 25,50 | 1.0 ~ 10 |
| Low Leakage | RL | Wide Temperature Range, Low Leakage Current | -55 ~ 105 | 1000 | 10 ~ 50 | 0.1 ~ 330 |
|  | LL | Standard, Low Leakage Current | -40 ~ 85 | 2000 | 10 ~ 63 | 1.0 ~ 4700 |
|  | LS | Height 7mm, Low Leakage Current | -40 ~ 85 | 2000 | 6.3 ~ 63 | 0.1 ~ 100 |
|  | LE | Height 5mm, Low Leakage Current | -40 ~ 85 | 1000 | 4 ~ 50 | 0.1 ~ 100 |
|  | TH | For TV Vertical Oscillator Circuits | -40 ~ 85 | 2000 | 16 ~ 50 | 1.0 ~ 100 |
|  | TB | For Timer Circuits | -40 ~ 85 | 2000 | 25 ~ 100 | 1.0 ~ 470 |
| Audio Use | AU | Standard, For Audio | -40 ~ 85 | 2000 | 6.3 ~ 100 | 0.1 ~ 4700 |
| Photo Flash | TF | Stndard, For Photo Flash | -20 ~ 55 |  | 260 ~ 330 | 60 ~ 180 |
| Ballast | BA | For Ballast Applications, Smaller Case Size | -40(-25) ~ 105 | 2000 | 160 ~ 450 | 1 ~ 220 |
|  | RH | For Ballast Applications | -40(-25) ~ 105 | 5000 | 160 ~ 450 | 10 ~ 150 |
|  | BL | For Ballast Applications, Long Life | -25 ~ 105 | 10000 | 160 ~ 450 | 6.8 ~ 150 |
|  | BW | For Ballast Applications, High Temp | -40(-25) ~ 125 | 2000 ~ 3000 | 160 ~ 450 | 1 ~ 47 |
| Charger | CH | For Charger, Adapter | -25 ~ 85 | 2000 | 400,450 | 2.2 ~ 68 |

5. Large Aluminum Electrolytic Capacitors

| Classification | Series | Applications | Operating Temperature Range (ĄÉ) | Load Life Time (hours) | Voltage Range (VDC) | Capacitance Range (§Ţ) |
| --- | --- | --- | --- | --- | --- | --- |
| SNAP-IN Terminal | HC | Standard | -40(-25) ~ 85 | 2000 | 6.3 ~ 500 | 56 ~ 100000 |
|  | HJ | Miniaturized | -40(-25) ~ 85 | 3000 | 160 ~ 450 | 56 ~ 3300 |
|  | HE | Standard, Wide Temperature Range | -40(-25) ~ 105 | 2000 | 6.3 ~ 450 | 47 ~ 68000 |
|  | HK | Miniaturized, Wide Temperature Range | -40(-25) ~ 105 | 3000 | 160 ~ 450 | 47 ~ 2700 |
|  | HU | Long Life (5000hours) | -40(-25) ~ 105 | 5000 | 200 ~ 450 | 47 ~ 1500 |
|  | HT | 15mm, Miniaturized | -40(-25) ~ 105 | 3000 | 160 ~ 450 | 39 ~ 390 |
|  | HB | High Temperature Range, For 125ĄÉ Use | -40 ~ 125 | 1000 | 10 ~ 200 | 100 ~ 15000 |
|  | QB | Withstanding Overvoltage | -25 ~ 105 | 2000 | 200,400 | 47 ~ 1000 |
| LUG TERMINAL | LM | For General Use | -40(-25) ~ 85 | 2000 | 16 ~ 450 | 68 ~ 150000 |
| Screw Terminal | SM | Standard | -40(-25) ~ 85 | 2000 | 16 ~ 450 | 220 ~ 680000 |
|  | CU | Standard, Wide Temperature Range | -40 ~ 105 | 2000 | 16 ~ 400 | 220 ~ 470000 |
|  | GT | For Inverter Circuits | -25 ~ 85 | 2000 | 350 ~ 450 | 180 ~ 10000 |
|  | GF | For Inverter Circuits, Long Life(5000hours) | -25 ~ 85 | 2000~5000 | 350 ~ 500 | 1000 ~ 12000 |
|  | EV | For Inverter Circuits, Wide Temperature Range | -25 ~ 105 | 5000 | 400 ~ 450 | 2200 ~ 6800 |
| Audio Use | AM | For Audio Equipment | -40 ~ 85 | 2000 | 16 ~ 100 | 470 ~ 33000 |
| Photo Flash | DF | For Photo Flash | -20 ~ 55 |  | 330,360 | 150 ~ 1500 |
| Air-Conditioning | AR | For Inverter Air-Conditioning System | -25 ~ 85 |  |  |  |
| Special Type | HP | Horizontal Mount Type | -40 ~ 105 | 2000 | 200,400 | 68 ~ 1000 |