### RightMark Audio Analyzer test report

| Testing device | Lexicon Alpha |
| --- | --- |
| Sampling mode | 24-bit, 48 kHz |
| Interface | ASIO |
| Testing chain | External loopback (line-out - line-in) |
| RMAA Version | 6.4.1 |
| 20 Hz - 20 kHz filter | ON |
| Normalize amplitude | ON |
| Level change | -5.4 dB / -5.4 dB |
| Mono mode | OFF |
| Calibration singal, Hz | 1000 |
| Polarity | correct/correct |

#### Summary

| Frequency response (from 40 Hz to 15 kHz), dB | +0.02, -0.91 | Good |
| --- | --- | --- |
| Noise level, dB (A) | -84.9 | Good |
| Dynamic range, dB (A) | 85.4 | Good |
| THD, % | 0.037 | Good |
| THD + Noise, dB (A) | -66.2 | Average |
| IMD + Noise, % | 0.053 | Good |
| Stereo crosstalk, dB | -68.3 | Good |
| IMD at 10 kHz, % | 0.214 | Average |
| General performance |  | Good |

#### Frequency response

|  | Left | Right |
| --- | --- | --- |
| From 20 Hz to 20 kHz, dB | -1.78, +0.08 | -1.87, +0.02 |
| From 40 Hz to 15 kHz, dB | -0.83, +0.08 | -0.91, +0.02 |

#### Noise level

|  | Left | Right |
| --- | --- | --- |
| RMS power, dB | -83.0 | -83.6 |
| RMS power (A-weighted), dB | -86.5 | -83.4 |
| Peak level, dB FS | -67.3 | -67.6 |
| DC offset, % | -0.0 | -0.0 |

#### Dynamic range

|  | Left | Right |
| --- | --- | --- |
| Dynamic range, dB | +82.2 | +83.5 |
| Dynamic range (A-weighted), dB | +86.8 | +83.9 |
| DC offset, % | +0.00 | +0.00 |

#### THD + Noise (at -3 dB FS)

|  | Left | Right |
| --- | --- | --- |
| THD, % | +0.0397 | +0.0344 |
| THD + Noise, % | +0.0421 | +0.0355 |
| THD + Noise (A-weighted), % | +0.0524 | +0.0454 |

#### Intermodulation distortion

|  | Left | Right |
| --- | --- | --- |
| IMD + Noise, % | +0.0546 | +0.0508 |
| IMD + Noise (A-weighted), % | +0.0475 | +0.0489 |

#### Stereo crosstalk

|  | Left | Right |
| --- | --- | --- |
| Crosstalk at 100 Hz, dB | -54 | -54 |
| Crosstalk at 1000 Hz, dB | -68 | -67 |
| Crosstalk at 10000 Hz, dB | -57 | -56 |

#### IMD (swept tones)

|  | Left | Right |
| --- | --- | --- |
| IMD + Noise at 5000 Hz, | 0.1579 | 0.1620 |
| IMD + Noise at 10000 Hz, | 0.1830 | 0.1878 |
| IMD + Noise at 15000 Hz, | 0.2945 | 0.3017 |